INSPECTAIR

Configuration Management

# 1. Versionskontrolle

Tool: Git

Repository: GitHub (privat)

Branching: main + feature branches

# 2. Entwicklungsumgebung

| Tool | Version / Details |
| --- | --- |
| IDE | Visual Studio Code |
| Build-System | PlatformIO |
| Framework | Arduino (espressif32 Platform) |
| Board | esp32-s3-devkitc-1 |
| Upload Speed | 921600 Baud |
| Monitor Speed | 115200 Baud |

# 3. platformio.ini

[env:esp32s3]

platform = espressif32

board = esp32-s3-devkitc-1

framework = arduino

monitor_speed = 115200

upload_speed = 921600

lib_deps =

lovyan03/LovyanGFX@^1.1.12

lvgl/lvgl@^8.3.0

sensirion/Sensirion Core@^0.6.0

sensirion/Sensirion Gas Index Algorithm@^3.2.2

adafruit/Adafruit AHTX0@^2.0.5

wifwaf/MH-Z19@^1.5.4

fu-hsi/PMS Library@^1.1.0

# 4. Hardware-Konfiguration

Alle Pin-Definitionen sind zentral in config.h definiert:

// Display (SPI)

#define TFT_CS    14

#define TFT_DC    13

#define TFT_MOSI  11

#define TFT_SCLK  12

#define TFT_RST   10

#define TFT_BL    21  // via MOSFET

// I2C (AHT20 + SGP40)

#define I2C_SDA   8

#define I2C_SCL   9

// UART Sensoren

#define CO2_RX    16

#define CO2_TX    15

#define PMS_RX    18

#define PMS_TX    17

#define RADAR_RX  6   // via Level Shifter!

#define RADAR_TX  7

#define RADAR_OUT 15

# 5. Dokumenten-Versionen

| Dokument | Version | Datum | Autor |
| --- | --- | --- | --- |
| Requirements Specification | 1.0 | Jan 2026 | Team |
| Design Description | 1.0 | Jan 2026 | Team |
| Test Specification | 2.0 | Jan 2026 | Maxi |
| Schedule | 1.0 | Jan 2026 | Team |
| Risk & Opportunities | 1.0 | Jan 2026 | Team |
| Coding Rules | 1.0 | Jan 2026 | Maxi |

— Ende des Dokuments —